PRE-MAINTENANCE REPORT OF THE SATATION LOCATED AT NYAMASHEKE, KARONGI AND NGORORERO DISTRICTS

- INTRODUCTION

This report presents the initial findings on the automatic weather stations located in Nyamasheke, Karongi and Ngororero Districts. Therefore, technicians are advised to take these observations into account when planning maintenance activities. The report is based on data collected from July 1st, 2025, to October 10th, 2025.

- FINDINGS

  - KARONGI DISTRICT

- GISHYITA AWS

- Reporting status: up to date

- Relative Humidity: from 2025-09-15 09:30:00 up to date humidity is always 0%.

- Battery: Battery drops up to 0V.

- Missing data: 48 days, 4 hours, 0 minutes; during night hours did not reports (check the details in provided excel file).

- NZARATSI AWS

- Data are not available in Web monitor or any platform

- ARGs

| SN | Station | Last communication | Issue |
| --- | --- | --- | --- |
| 1 | Murambi | Up to date | No issue |
| 2 | Murundi | Up to date | No issue |
| 3 | Bwishyura | Up to date | No issue |

  - NYAMASHEKE DISTRICT

- WISUMO AWS

- Reporting status: up to date

- Rain: Records only total rainfall of 4.6 mm from 1/07/2025 to 24/09/2025.

- Missing data: 65 days, 5 hours, 40 minutes; during night hours do not reports (check the details in provided excel file).

- KANJONGO AWS

- Reporting status: Does not report (No any available data).

- NYAMASHEKE AWS

- Reporting status: up to date

- Rain: It is not in harmony with manual station (on 1/7/2025 from 15:00 to 15:10 records 52 mm while on same day manual station records 0 mm).

  - RUTSIRO DISTRICT

- BUGARURA AWS

- Reporting status: up to date

- Rain: from 2025-10-01 00:45:00 started to rerecords suspected rainfall (from 01/10/2025 to 08/10/2025 records total rainfall of 2015.2mm).

- Wind direction data: have a decimal point.

- Missing data: 12 days, 14 hours, 35 minutes (check the details in provided excel file).

- NYABIRASI AWS

- Reporting status: up to date

- Air temperature data: Are not available.

- Battery data: Varying between 0.2V and 1.3V (but it reports consistency)

- Missing data: 12 days, 14 hours, 35 minutes (check the details in provided excel file).

- ARGs

| SN | Station | Last communication | Issue |
| --- | --- | --- | --- |
| 1 | Mushubati | Not reporting | Not known |
| 2 | Ruhango | Up to date | No issue |
| 3 | Kivumu | Up to date | No issue |

Prepared by:

Christian Bikorimana

Ag.Data Quality Control Officer